SV 650R By Greg Johnson

Materials list:

- 1 Set of Shark Skinz 							$1,000.

- Wind screen								              65.

- Mounting Brackets							            110.

- 1 Set of Pilot Projector Beam lights					                30.

- 1 piece of 1/8" thick plexiglass 2' x 2'					                20.

- Fiberglass resin and cloth 						                10.

- 3 Pieces of aluminum rod 1/4" diameter 				                12.

- Set of 41mm clip on handle bars 	off of zx6R                                                                      120.

- 1 Set of 1" heat wrap for exhaust system 				                20.

- Exhaust system (your choice) 				                      500-700.

- Piece of 1" square aluminum tube.					                    5.

Tools list:

- Metric wrench and socket set.

- Assorted screw drivers

- Locking plyers

- X-acto knife with saw blade

- Sand paper (assorted grits)

- High heat air gun (used to remove paint and graphics)

- Insulated gloves (cooking mitts are best)

- Power drill with various bits and screw driver attachments

- Jig saw (reciprocating style)

- Metal file

- Power grinder for metal

- Metal saw (hand held will be fine )

- Race stand for the front and rear

Skills required:

- Basic mechanical knowledge

- Good knowledge of tools and operation

- Good basic body work skills

- Above average paint skills (can hire this stage out to a professional)

- Above average knowledge of electrical wiring

SV650R By Greg Johnson

The basic idea of the SV650R came from the Internet. I do a lot of surfing as I am a Web designer and love to visit motorcycle Web Sites. I came across one web page that peaked my interest: <ww.sv650.org>. This is a site that is dedicated to the Suzuki SV650. I saw some amazing pictures of a motorcycle with a gray bodywork that looked like a cross between the Bat Cycle and a sport bike. The more I searched the more I was convinced that this was the bike I wanted to build, only I wanted to take it off the track and put it on the street! So the process began! The first dilemma was to purchase an Sv650. This was no easy feat. No dealer had one in blue. If I was going to do it I wanted it to be blue ! I finally found one and purchased it . The first night I took it home I began the process by removing the bars from below the tail section that is used to attach a cargo net. Yes I took a saw to a brand new bike with no miles on it . I never said I was smart! The next step was to get rid of the up-right handle bars and the massive head light that was the size of a small cocaine producing country in Central America. The bike was fitted with a set of 41mm clip on race handle bars from a wrecked zx6R a friend of mine had. Fit like a glove. Now the fun part began! The gauges were removed and the rear tail section was removed.

The next step was to order the body work from SharkSkinz < It is necessary to order a set of brackets to mount the body work with and a wind screen from them as well. I called Daphney and was told to my horror that I was 15th on the waiting list for a set of the bodywork. I have never been one to wait on anything so this was the worst news I could have received. I waited none the less until finally the guys in the little brown truck arrived. I got my package and kissed my wife goodbye!(she knew she would not see me for a few weeks) I went to my garage and unloaded the box. This was the finest set of bodywork I have ever seen not a flaw in it. The bodywork is pre-drilled and matches up perfectly when the brackets are installed on the bike . The brackets come with pop rivets (do not use them!) You will be better off using sheet metal screws that can be tightened or removed. If you use the COLOR instruction sheet you can not go wrong it is great ! The brackets installed I started hanging bodywork. This was a breeze! Every screw was right where there was a hole pre-drilled in the body work. Everything matched up perfectly. The tail section had to be cut to accommodate the rear tail lamp assembly . This was done using a small hand held (jig saw). A reciprocating saw of any kind would work if the saw blade was small enough. The rear sub frame had to be ground down to prevent wearing through the solo tail piece. The top edge needs to be ground down at the point of the back end top section of the sub frame. Adding a piece of foam ruber ½" thick to the underside of the rear tail section will insure safety and cut down on movement.    With the bodywork installed the bike really started to take shape. I now had to come up with some headlamps to use. This proved to be a difficult task as no one made them so the choices are few and far between. I had the fortune to be raised with a father who was a jack of all trades (and master of none). He painted cars for a living and taught me a lot about body work and how to fabricate anything. I started with the basic idea of a R1 or an R6 shape but different enough to give them some uniqueness. I made a template of the shape and transferred it to the bodywork . I then purchased some plexiglass 1/8" thick and using a high heat air gun shaped it to the bodywork. This is tricky and dangerous because of the heat. It is best if you use a cooking mitt to shape the plexiglass with. Using the template I drew the shape on the plexiglass and cut it out with a special saw blade in the (jig saw). I now had my lense for my light. The next step was to cut out the shape from the bodywork. (Yes cut into a $1000 set of Sharkskinz (again I never said I was smart).

I used an Xacto or (hobby ) knife with a small saw blade to cut the shape out . The blade is very thin and you get a narrow gap between the light and the bodywork. Do not throw this piece away!

The pieces that are removed are the backing plates for the lights. In the backing plates I cut an elliptical hole in the center. This will be the hole where the tube is grafted on the back to hold the Pilot projector beam light. The back of the light consists of three parts. The backing plate, the tube and the light element.    To make the tube and graft it onto the backing plate start with a piece of thick wax paper and roll it up to make a tube . Fit the tube through the elliptical hole in the backing plate. Tape it on the front side that faces out the front of the bike. Now test fit the projector beam light to see if it will fit. It was necessary to make a template of the shape of the light and cut it out to be placed in the middle of the tube to hold the shape while the fiberglass resin dries. Be sure to put the wax side of the paper out . This will keep the resin from sticking to the paper and allow you to remove it from the tube when the resin hardens. With the tube attached to the backing plate cut approx. 3" back . Cut and test fit the light now . If you will take the front lense and mask off an elliptical shape (larger than the one for the tube by 1/4")and spray the back side of it black it will mask    light from areas you don’t want light to come through.

Spray the backing plate and tube flat black inside the tube and outside to eliminate light leakage.

To secure the lense to the backing plate you will need some high temp silicone used in engine building . Put a liberal coating on the back side of the lense and be sure not to get it on the clear section. Adhere it to the backing plate and the silicon will act as a bonding agent and weather sealant . I also used it to secure the Pilot projector beam lights in the tubes . The next step is to mount the lights to the front fairing bracket. Take a piece of aluminum tubing 1" square approx. 14 to 16 inches long (measure to fit the bodywork ). In the center of the aluminum bar mark a cut 1/8th inch down with a metal saw and notch out to create a channel that sets down over the fairing bracket. See picture for diagram. Secure with two sheet metal screws. (Do not weld yet you may need to adjust the angle of the bar to align the lights).    Now mount the lights to the brackets and fit the front section of the bodywork on the bike to see how the alignment looks . Slide the headlamp lense assembly over the Pilot projector beam lights to test alignment . You are almost home now the rest is a simple trial and error for alignment . I have found it best to aim the lights at a wall about 6 ft ahead of the bike and align the lights together about 3.5 feet off the groung this will give a good field of vision and will not br too high to bother on-coming drivers. To wire the lights just pair the ground wires together and pair the hot wires together to create one ground and one hot. These will be fitted with an electrical end that will plug into the existing headlamp electrical plug. If you have an SV650S model I believe you have two of these and you do not need to combine the wires of the Pilot projector beam lights . Use a light tester to find a ground and the high beam setting plug . This way the head lamps scan be turned on or off. You can also get the flush mount marker lights from most magazines that mount to the bodywork for turn signals. I opted to not add these and use hand signals in traffic . The mirror option is up to the discretion of the owner . I do not use mirrors (not really concerned with what I have all ready passed!). If you choose to use turn signals just wire to the factory plugs cut the wire and splice in the new wire to get the ground and hot wires.    The SharkSkinz came with a upper fairing bracket that is a T welded to the center of the front fairing bracket. I choose to cut this T off at the top of the T and bent it back toward the front to make a bracket to mount the gauges to. Drill a hole and mount the gauges to the flat bar that you bent and adjust the angle as needed.

To fabricate a upper fairing bracket you will need someone who is familiar with welding and bending aluminum.

I found it best to job this out . The bracket I have created I am sending to Daphney at SharkSkinz this winter to allow them to measure it and copy it so they can offer it as a separate piece for their customers. The final job was to paint the bike and to do this I needed a paint to match . I contacted color-right in California the largest supplier of Japanese bike paint in the US to get the paint . It is not cheap about $80 in the US for a quart. I sprayed the bike and let it cure for 1 month before painting my graphics on it. To do this I used a hand held spray can KRYLON paint from a local hardware store. Then I trimmed it with a silver tape to cover the edges of the paint. The wheels and front forks were sprayed with an enamel paint also out of a spray can .    You must remove the wheels and tires to spray the wheels or you will get paint everywhere. To say the least it was a MAJOR undertaking but one well worth the time and money invested to create a bike that no matter where you go will always gather a crowd and has been a conversation piece at every motorcycle event I have attended. If anyone wishes to contact me they can do so at my

E-mail address  or they can visit my web page at <

Thankfully

Greg Johnson